UC-S2-01

# Investigate and Classify Underground Cable Fields

Sprint 2 Deliverable: Priority Field Matrix — field classification and data understanding

Dataset: Underground Cable Synthetic data(2).csv  |  5,000 records  |  21 fields

Outcome All 21 fields in the supplied synthetic underground-cable dataset are classified by role, type, unit/category and purpose. The three requested priority fields — asset_age_years, groundwater_indicator and corrosion_indicator — are explicitly treated as priority target/predictor fields. The dataset is complete (0 missing values), so it is suitable as the known reference version before controlled masking.

# 1. Dataset overview

| Check | Result | Interpretation |
| --- | --- | --- |
| Rows | 5,000 | One record per synthetic underground-cable asset |
| Columns | 21 | Identifier, engineering, spatial and environmental fields |
| Missing values | 0 | Reference dataset is fully complete |
| Duplicate rows | 0 | No full-row duplicates |
| Unique asset IDs | 5,000 | One unique asset_id per row |
| Feeders | 60 | FDR_001–style grouping identifiers |
| Installation year | 1950–2026 | asset_age_years spans 0–76 years |

# 2. Field classification

| Field | Primary role | Type | Unit / categories | Purpose / interpretation |
| --- | --- | --- | --- | --- |
| asset_id | Identifier | Text | 5,000 unique IDs | Traceability and joins. Do not use as a predictive signal. |
| feeder_id | Grouping / predictor | Categorical | 60 feeder IDs | Groups assets for similar-asset or feeder-level analysis. |
| rated_voltage_kv | Predictor | Integer | 11 or 33 kV | Electrical rating; possible engineering segmentation variable. |
| installation_year | Derived source / target / predictor | Integer | Year: 1950–2026 | Direct source for age. If known, derive asset_age_years instead of imputing it. |
| asset_age_years | PRIORITY target / predictor | Integer | Years: 0–76 | Core ageing variable for reliability/Weibull-style analysis. Exact relation: 2026 − installation_year. |
| cable_length_m | Predictor | Numeric | Metres: 15–1,800 | Physical exposure/extent; also related to joint_count in the synthetic data. |
| insulation_type | Predictor / possible target | Categorical | XLPE, PILC | Cable technology/ageing characteristic; strong age separation in this synthetic dataset. |
| conductor_material | Predictor / possible target | Categorical | Aluminium, Copper | Cable construction characteristic; associated with age in the synthetic data. |
| conductor_size_mm2 | Predictor | Integer | mm²: 50–630 | Engineering specification and possible asset-type discriminator. |
| core_count | Predictor | Integer | 1 or 3 cores | Construction characteristic; shows a notable age relationship in the synthetic data. |
| installation_method | Predictor | Categorical | Duct/Conduit, Direct Buried | Environmental/protection context; useful for corrosion-related modelling. |
| latitude | Enrichment key / spatial predictor | Numeric | Decimal degrees | Links assets to spatial/environmental data; midpoint of route geometry. |
| longitude | Enrichment key / spatial predictor | Numeric | Decimal degrees | Links assets to spatial/environmental data; midpoint of route geometry. |
| joint_count | Predictor / possible target | Integer | 0–9 joints | Reliability-relevant construction/repair characteristic. |
| route_geometry_wkt | Spatial source / enrichment | Text geometry | 5,000 LINESTRINGs | Route geometry for spatial derivation; redundant with midpoint latitude/longitude for basic location. |
| cable_type_raw | Raw composite / derivation source | Text | 24 raw descriptions | Encodes size, core count, material, insulation and voltage; useful for parsing structured fields. |
| groundwater_depth_m | Enrichment / predictor | Numeric | Metres: 1.16–11.43 | Environmental measurement supporting groundwater exposure interpretation. |
| groundwater_exposure_score | Derived enrichment / predictor | Numeric | Score: 0.240–0.943 | Continuous groundwater-exposure signal; strongly inversely related to groundwater depth. |
| groundwater_indicator | PRIORITY target / predictor / derived | Categorical | Low, Medium, High | Priority environmental field. Closely tied to groundwater_exposure_score. |
| corrosion_exposure_score | Derived predictor | Numeric | Score: 0.121–0.984 | Continuous corrosion exposure signal; related to groundwater exposure and installation method. |
| corrosion_indicator | PRIORITY target / predictor / derived | Categorical | Low, Medium, High | Priority deterioration/exposure field. Closely tied to corrosion_exposure_score. |

# 3. Important structure and leakage findings

Exact age derivation: asset_age_years = 2026 − installation_year for all 5,000 records. If installation_year is available, age should be derived rather than statistically imputed.

Indicator leakage risk: groundwater_indicator is a categorical form of groundwater_exposure_score, and corrosion_indicator is a categorical form of corrosion_exposure_score. The observed category boundaries are approximately around 0.4 and 0.7; rounded scores make exact boundary cases ambiguous.

Composite-text leakage risk: cable_type_raw reproduces conductor size, core count, conductor material, insulation type and voltage in all 5,000 rows checked. If one of those structured fields is masked while cable_type_raw remains visible, a model can recover the answer by parsing the text rather than learning a genuine imputation relationship.

Spatial redundancy: latitude/longitude match the midpoint of route_geometry_wkt to within roughly 5.0e-07 degrees. Use route geometry for spatial processing, but avoid treating all three fields as independent evidence.

Environmental relationship: groundwater_depth_m and groundwater_exposure_score have correlation -0.900; corrosion_exposure_score and groundwater_exposure_score have correlation 0.673.

Installation relationship: Mean corrosion score is 0.623 for Direct Buried cables versus 0.536 for Duct/Conduit cables.

# 4. Recommended use of the three priority fields

| Priority field | Recommended treatment |
| --- | --- |
| asset_age_years | Use as a priority imputation target only when installation_year is unavailable or also masked. Otherwise derive it deterministically. Candidate predictors include insulation_type, conductor_material, core_count, joint_count, feeder_id and other engineering fields. |
| groundwater_indicator | If groundwater_exposure_score is present, derive/classify the indicator rather than impute it. For a genuine imputation test, mask the score at the same time and use groundwater_depth_m, location and installation_method as candidate predictors. |
| corrosion_indicator | If corrosion_exposure_score is present, derive/classify the indicator rather than impute it. For a genuine imputation test, mask the score and use groundwater variables, installation_method, location and other asset attributes. |

UC-S2-01 completion status Done when: all relevant cable fields are classified and documented. This document classifies all 21 fields and identifies direct derivations/redundancies that must be controlled during later imputation experiments.